QUALITY CHECKLIST FOR PLANS EXAMINERS :

STAND NO : .....................TOWNSHIP:...............................................................

DOCUMENTATION :

- Check if title deed is submitted.

- Check for restrictions in deed.

- If not registered, a draft deed with an attorneys letter, confirming the transfer to be supplied.

- Zoning and SG Diagrams must be supplied.

- Check if the siteplan corresponds with the SG Diagram. Check if zoning permits the use shown on the plans.

- SANS 10400 FORMS 1-2 must always be completed by the owner, architect and competent persons (engineers, specialists)

- SACAP Registration form must always be submitted.

- Always make sure that the owners’ details and signature are on the plans.

- Always make sure that the architect/draughtsman/architectural technologists details are on the plans.

- Check that a registration number is supplied by the Architect, or registered person.

- Additional documentation may be necessary depending on the application eg . Fire rationals, building line relaxations sdps etc.

BUILDING PLANS :

- 2 sets of plans to be supplied (1x colour, 1x black).

- Also, the preferred paper size is A4,A3,A1

- Only large projects should be submitted in A0 size. This is not compulsory, but should be encouraged.

- The plans should be legible enough to read.

- The scale cannot exceed 1:100 for floorplans, and 1 : 200 for elevations.

SITEPLANS :

- Siteplans can be in modules from 1: 100, 1: 200, 1: 250, 1 : 500, 1 :750 OR 1 : 1000. Very seldom 1 : 1500.

- Building lines to be shown, and if encroached the customer must be advised to relax prior to submission

- All boundaries must be clearly shown, and dimensioned. Cross check with SG Diagram.

- All existing structures to be shown with clearly dimensioned distances from all boundaries

- All new structures must be coloured in and distances from boundaries and building lines to be shown on.

- Any servitudes or existing municipal services on the site to be shown. Nothing to encroach into servitudes

- Municipal sewer connection point to be indicated, together with existing and new drainage connecting to such.

- North point to be provided and scale of drawing.

- All proposed demolition work to be shown

- Street name and all street furniture to be shown outside the site.

- All adjacent stands to be shown in dotted line, together with adjacent stand numbers

- A full area schedule must be supplied indicating the ffg :

STAND AREA…………sqm

EXISTING FLOOR AREA…………sqm

NEW ADDITIONAL FLOOR AREA…………sqm

TOTAL FLOOR AREA…………..sqm (ex + new)

TOTAL F.A.R……………sqm

TOTAL COVERAGE……….sqm + …….%

FLOORPLANS :

- All new work to be coloured in, and hatched on floorplans, including drainage.

- Floorplans must be drawn in 1:100 scale, unless the project is of a large proportion.

- All rooms/spaces to be designated and labelled.

- Full dimensions to all areas must be shown

- Existing structures must be differentiated from new, by not being coloured in.

- All demolished structures to be indicated in dotted lines.

- Section lines and direction of cutting planes to be shown correctly.

- Full details of drainage, including existing and new sewer lines.

- All sanitary-ware must be shown to scale, and connection points.

- Staircases to be indicated correctly with stair notes (no. of risers and treads)

- All doors and windows/glazing areas to be shown in proper scale.

- Full floorplan/ layout must be submitted for additions. No partial/ cut out floorplans must be accepted.

SECTIONS :

- Section lines to be clearly shown, with correct direction.

- All new work to be coloured in.

- All floors to ceiling heights must be dimensioned.

- All floors to underside of lintol height to be dimensioned (windows and doors)

- Correct scales to be shown (1:100, 1:50, 1:25, 1: 20 etc.)

- All construction notes to be shown. (Roof notes, slab notes, and surface beds, and foundations)

- Full stair detail sections to be shown if required.

- Heights to boundary walls, or terrace walls must be indicated where necessary

- Natural ground levels to be shown (NGL)

- Finished floor levels to be indicated (FFL)

ELEVATIONS :

- All new work to be coloured in.

- Make sure that all necessary elevations are shown. (North, South, East, West)

- Correct scale of elevations must be indicated (1:50, 1:100, 1:200 only)

- All external finishes to be noted on elevations

- Natural ground levels, and Finished floor levels to be shown on each elevation.

- Drainage to be shown on elevation.

- All heights of boundary walls and terraces to be dimensioned.

- Distances from boundaries to be indicated on all elevations

Please note that the above mentioned requirements are only a checklist for submission purposes, and does imply an immediate approval of building plans. A full examination after submission may require additional information.

CHECKED BY PLANS EXAMINER:........................................................................................................

SIGNED :……………………………………………………………..

SIGNED BY CUSTOMER :……………………………………..

DATE :…………………................................................